Lesson 05 Report

What Is Initial Phase?

Project: Beamforming From First Principles Using MATLAB

Chapter: 01 – Fundamentals of Waves

Lesson: 05 – What Is Initial Phase?

Version: 1.0.0

Question

What is the initial phase of a sinusoidal waveform?

If two sinusoidal signals have exactly the same amplitude and frequency, why might they not start at the same point on the time axis?

How does the initial phase influence the waveform without changing its amplitude or frequency?

Objective

After completing this lesson, students will be able to:

- Understand the physical meaning of initial phase.

- Explain the role of phase in a sinusoidal waveform.

- Distinguish initial phase from amplitude, frequency, and period.

- Express phase in both degrees and radians.

- Generate sinusoidal signals with different initial phases using MATLAB.

- Compare waveforms with different initial phases in the time domain.

- Understand why phase is one of the most important concepts in communication systems, radar, and beamforming.

Background

So far in this chapter, we have introduced three important characteristics of a sinusoidal waveform:

- Amplitude

- Frequency

- Period

These parameters describe the size of the waveform, how fast it oscillates, and the duration of one complete cycle. However, another important question remains:

Where does the waveform begin?

Two sinusoidal signals may have identical amplitudes, identical frequencies, and identical periods, yet they may not start at the same point in time.

The parameter that determines the starting position of a sinusoidal waveform is called the initial phase. Initial phase specifies the value of the waveform at the initial instant, usually at

t=0.

Changing the initial phase shifts the waveform horizontally along the time axis without altering its shape.

Unlike amplitude, the initial phase does not change the maximum or minimum values of the waveform. Unlike frequency, it does not change how rapidly the waveform oscillates. Instead, it changes only where the oscillation begins. This idea may appear simple, but it is one of the most fundamental concepts in communication engineering. Phase differences determine how multiple waves combine. Two waves with the same amplitude and frequency may reinforce one another if they are in phase, or cancel each other if they are out of phase. For this reason, phase is central to many engineering disciplines, including:

- Wireless communications

- Radar systems

- Antenna arrays

- Beamforming

- Navigation systems

- Signal synchronization

- Digital signal processing

Understanding phase is therefore an essential prerequisite for studying beamforming algorithms, where signals arriving at different antennas differ primarily in their phases.

Mathematics

The mathematical expression of a sinusoidal signal is

x(t)=Asin⁡(2πft+ϕ)

where

- Ais the amplitude,

- fis the frequency,

- tis time,

- ϕis the initial phase.

The phase angle shifts the waveform horizontally.

For example,

ϕ=0∘

produces the standard sine wave.

When

ϕ=90∘

the waveform begins at its maximum value because

sin⁡π2=1.

Similarly,

ϕ=180∘

produces

-sin⁡(2πft),

which starts in the opposite direction.

These examples demonstrate that the phase changes the starting position of the waveform while leaving its amplitude, frequency, and period unchanged.

Phase can be expressed in either degrees or radians.

The relationship between these units is

180∘=π rad.

In engineering calculations, radians are generally preferred because they naturally arise in mathematical analysis and signal processing.

MATLAB Implementation

The objective of this MATLAB simulation is to investigate the effect of initial phase on a sinusoidal waveform while keeping all other signal parameters constant.

To isolate the influence of phase, the following parameters remain unchanged throughout the simulation:

- Amplitude

- Frequency

- Sampling frequency

- Simulation duration

Only the initial phase is varied.

Three sinusoidal signals are generated using the following initial phase angles:

- 0°

- 45°

- 90°

Before generating the signals, the phase values are converted from degrees to radians because MATLAB trigonometric functions operate in radians.

The reusable function generateSineWave.m is then used to generate each waveform.

Finally, all three signals are plotted on the same figure for direct comparison.

This approach follows an important engineering principle adopted throughout this educational project:

When studying one parameter, keep all other parameters constant.

By changing only the initial phase, students can clearly observe its effect without confusing it with changes caused by amplitude or frequency.

Visualization

The generated figure displays three sinusoidal waveforms with identical amplitudes and frequencies but different initial phases.

The horizontal axis represents time (seconds), while the vertical axis represents signal amplitude.

Several important observations can be made:

- All three waveforms have exactly the same amplitude.

- All three waveforms have the same frequency.

- All three waveforms have the same period.

- Only the starting position of each waveform is different.

The waveform with an initial phase of 0° begins at the origin.

The waveform with an initial phase of 45° begins partway through its oscillation.

The waveform with an initial phase of 90° starts at its maximum value.

Although the waveforms appear shifted relative to one another, they remain identical in every other aspect.

This demonstrates that the initial phase controls where the waveform begins, not how large it becomes or how rapidly it oscillates.

Discussion

The simulation confirms that the initial phase is responsible for shifting a sinusoidal waveform along the time axis.

Unlike amplitude, which changes the vertical scale of the waveform, or frequency, which changes the oscillation rate, the initial phase affects only the waveform's starting position.

This distinction is essential because many engineering systems rely on precise phase relationships rather than changes in amplitude or frequency.

One common misconception is that a phase shift changes the waveform itself. In reality, the waveform remains mathematically identical; only its position along the time axis changes.

This concept becomes particularly important when multiple sinusoidal signals are combined.

If two signals have the same amplitude and frequency but different phases, they may reinforce each other (constructive interference) or partially or completely cancel each other (destructive interference).

These interference effects are fundamental to many engineering applications, including:

- Wireless communications

- Radar systems

- Medical ultrasound imaging

- Sonar

- Acoustic signal processing

- Antenna arrays

Among these applications, beamforming is one of the most significant.

Beamforming does not rely on changing the frequency or amplitude of individual antenna signals. Instead, it manipulates their relative phases so that electromagnetic waves combine constructively in desired directions and destructively in undesired directions.

For this reason, the concept introduced in this lesson serves as one of the most important foundations for the remainder of this educational project.

By understanding how the initial phase shifts a waveform, students are taking their first step toward understanding how antenna arrays steer electromagnetic energy without physically moving the antennas.

Exercises

Exercise 1

Generate three sinusoidal signals with the following initial phases:

- 0°

- 45°

- 90°

Keep the amplitude and frequency constant.

Compare the resulting waveforms and explain the differences.

Exercise 2

Generate four sinusoidal signals with the following initial phases:

- 0°

- 90°

- 180°

- 270°

Describe the starting point of each waveform.

Exercise 3

Create two sinusoidal signals having:

- The same amplitude

- The same frequency

- Different initial phases

Explain why the signals have identical shapes but different starting positions.

Exercise 4

Convert the following phase angles from degrees to radians.

| Degrees | Radians |
| --- | --- |
| 30° |  |
| 45° |  |
| 60° |  |
| 90° |  |
| 180° |  |
| 270° |  |
| 360° |  |

Verify your answers using MATLAB.

Exercise 5

Without changing the frequency or amplitude, investigate how increasing the initial phase affects the waveform.

Explain why changing the phase does not change the waveform's amplitude or period.

Key Takeaways

After completing this lesson, the following concepts should be clear:

- Initial phase determines the starting position of a sinusoidal waveform.

- Phase is measured in degrees or radians.

- MATLAB trigonometric functions use radians.

- Changing the initial phase does not change:

  - Amplitude

  - Frequency

  - Period

- Changing the initial phase shifts the waveform horizontally along the time axis.

- Two signals with identical amplitudes and frequencies may still be different because of their phases.

- Relative phase differences determine how multiple waves combine.

- Phase is one of the fundamental concepts behind antenna arrays and beamforming.

Remember:

Amplitude determines how large a waveform is.

Frequency determines how fast it oscillates.

Initial phase determines where it begins.

References

The concepts presented in this lesson are based on standard textbooks in signal processing, communications, and electromagnetics.

Textbooks

- Alan V. Oppenheim and Alan S. Willsky, Signals and Systems, 2nd Edition, Pearson Education.

- Simon Haykin and Barry Van Veen, Signals and Systems, 2nd Edition, Wiley.

- John G. Proakis and Dimitris G. Manolakis, Digital Signal Processing: Principles, Algorithms, and Applications, 4th Edition, Pearson.

- Constantine A. Balanis, Antenna Theory: Analysis and Design, 4th Edition, Wiley.

- Steven W. Smith, The Scientist and Engineer's Guide to Digital Signal Processing, California Technical Publishing.

Conclusion

In this lesson, we introduced the concept of initial phase, which determines the starting position of a sinusoidal waveform without altering its amplitude, frequency, or period.

Through mathematical analysis and MATLAB simulation, we observed that changing the initial phase simply shifts the waveform along the time axis while preserving all of its other characteristics. By comparing several waveforms with identical amplitudes and frequencies but different phase angles, we isolated the effect of phase and demonstrated its purely horizontal influence on the waveform.

This lesson also established an important distinction between absolute phase and relative phase. While the initial phase describes the starting point of a single waveform, the difference in phase between multiple waveforms governs how they interact. Depending on their relative phases, waves can reinforce one another through constructive interference or weaken each other through destructive interference.

This principle is fundamental to many areas of engineering, particularly wireless communications, radar systems, digital signal processing, and antenna arrays.

Most importantly, this lesson marks the beginning of the conceptual path toward beamforming. Modern beamforming techniques rely on carefully controlling the relative phases of signals received or transmitted by multiple antenna elements. By introducing phase as a controllable parameter, students are now prepared to understand how antenna arrays manipulate electromagnetic waves to steer energy toward desired directions.

In the next lesson, we will study time delay, another mechanism that shifts a waveform in time. Students will discover that time delay and phase shift are closely related but are not identical concepts. Understanding this distinction is essential before introducing wave propagation, wavelength, steering vectors, and the mathematical foundations of beamforming.