Lesson 06 Report

What Is Time Delay?

Project: Beamforming From First Principles Using MATLAB

Chapter: 01 – Fundamentals of Waves

Lesson: 06 – What Is Time Delay?

Version: 1.0.0

Question

What is time delay?

If two sinusoidal signals have identical amplitudes, frequencies, and initial phases, why might one arrive later than the other?

How does a time delay affect the appearance of a waveform, and why is this concept fundamental to wave propagation and beamforming?

Objective

After completing this lesson, students will be able to:

- Understand the physical meaning of time delay.

- Define time delay as the difference in signal arrival time.

- Explain how time delay shifts a waveform along the time axis.

- Distinguish time delay from initial phase.

- Generate delayed sinusoidal signals using MATLAB.

- Interpret delayed waveforms in the time domain.

- Recognize the importance of time delay in communications, radar, and beamforming.

Background

In the previous lesson, we learned that the initial phase determines where a sinusoidal waveform begins at the initial instant. Although changing the initial phase shifts the waveform horizontally, it does not represent a physical propagation process.

In contrast, time delay is a physical quantity.

It describes the amount of time required for a signal to travel from one location to another or the difference in arrival times between two observations of the same waveform.

Consider a loudspeaker producing a sound wave. A listener standing one meter away hears the sound before another listener standing ten meters away. The waveform itself has not changed; rather, it arrives at different times because it must travel through space.

The same principle applies to electromagnetic waves.

When a radio transmitter emits a signal, the wave propagates at a finite speed. Receivers located at different positions observe the same waveform at different times.

This difference in arrival time is called the time delay.

Time delay is therefore not a property of the waveform itself but a consequence of wave propagation.

Changing the time delay does not alter the waveform's amplitude, frequency, or period. Instead, it shifts the entire waveform along the time axis by a fixed amount.

This concept is fundamental to engineering because nearly every system involving wave propagation experiences time delays, including:

- Wireless communication systems

- Radar

- Sonar

- Acoustic systems

- Medical ultrasound

- Satellite navigation

- Microphone arrays

- Antenna arrays

Among these applications, antenna arrays are especially important.

When an incoming electromagnetic wave reaches an antenna array, it does not arrive at every antenna element simultaneously. The element closest to the incoming wave receives the signal first, while the remaining elements receive it slightly later.

These small differences in arrival time are the physical origin of the phase differences exploited by beamforming algorithms.

Understanding time delay is therefore the first step toward understanding how antenna arrays determine the direction of arrival of a signal and steer beams electronically.

Mathematics

A delayed sinusoidal signal can be represented as

x(t)=Asin⁡ ⁣2πf(t-τ)+ϕ

where

- Ais the amplitude,

- fis the frequency,

- tis time,

- τis the time delay,

- ϕis the initial phase.

The quantity τis measured in seconds (s) and represents the amount of time by which the waveform is shifted.

When

τ=0,

the signal is observed without delay.

As the value of τincreases, the waveform moves toward the right on the time axis, indicating that it arrives later.

An important observation is that introducing a time delay does not change the amplitude, frequency, or period of the waveform.

It only changes when the waveform is observed.

This distinction is crucial because, although a delayed waveform may appear similar to a phase-shifted waveform, the two concepts originate from different physical principles.

The mathematical relationship between time delay and phase shift will be developed in the next lesson.

MATLAB Implementation

The objective of this MATLAB simulation is to demonstrate the effect of time delay on a sinusoidal waveform while keeping all other signal parameters unchanged.

To isolate the influence of time delay, the following parameters remain constant throughout the simulation:

- Amplitude

- Frequency

- Initial phase

- Sampling frequency

- Simulation duration

Only the time delay is varied.

Three sinusoidal signals are generated using the following delays:

- 0 ms

- 20 ms

- 40 ms

Instead of modifying the waveform directly, the simulation delays the signal by replacing the time variable twith t-τ, where τrepresents the time delay.

The delayed waveform is therefore generated according to

x(t)=Asin⁡ ⁣2πf(t-τ)+ϕ.

The reusable function generateSineWave.m is used to generate each waveform, while different delay values are applied before the signal is evaluated.

Finally, all delayed waveforms are plotted on the same figure for direct comparison.

This approach allows students to investigate the effect of propagation delay independently from other waveform parameters.

Visualization

The generated figure compares three sinusoidal waveforms having identical amplitudes, frequencies, and initial phases but different arrival times.

The horizontal axis represents time (seconds), while the vertical axis represents signal amplitude.

Several important observations can be made:

- All waveforms have the same amplitude.

- All waveforms have the same frequency.

- All waveforms have the same period.

- The only difference is the instant at which each waveform appears.

The waveform with 0 ms delay begins immediately.

The waveform with 20 ms delay appears later.

The waveform with 40 ms delay appears even later.

The shape of the waveform remains completely unchanged.

Only its position along the time axis is shifted.

This confirms that a time delay does not distort the waveform; it simply postpones its arrival.

The visualization also illustrates an important physical idea.

The delayed signals can be interpreted as observations of the same wave measured at different locations or at different moments in time.

Discussion

Time delay is one of the most fundamental concepts in wave propagation because no physical wave travels instantaneously.

Whether the wave is electromagnetic, acoustic, or mechanical, it always requires time to propagate from one point to another.

Consequently, two receivers located at different positions will generally observe the same signal at different times.

This difference in arrival time is called the time delay.

Unlike the initial phase introduced in the previous lesson, time delay is directly related to the physical propagation of energy through space.

The distinction between these two concepts is extremely important.

An initial phase specifies the mathematical starting point of a waveform at its source.

A time delay describes when the waveform reaches an observation point after propagating through a medium.

Although both phenomena appear as horizontal shifts in the time domain, their origins are fundamentally different.

This distinction becomes particularly important in antenna arrays.

Consider a plane electromagnetic wave approaching a linear array of antennas.

The antenna closest to the source receives the wave first.

The remaining antennas receive exactly the same waveform, but each receives it slightly later because the wave must travel an additional distance.

These tiny propagation delays are typically measured in nanoseconds.

Although extremely small, they contain valuable information about the direction from which the wave arrived.

Beamforming algorithms exploit these propagation delays to estimate the Direction of Arrival (DOA) and to electronically steer transmitted or received beams.

For this reason, time delay is not merely another waveform parameter.

It is the physical mechanism that ultimately enables antenna arrays to determine direction, focus energy, suppress interference, and improve communication performance.

In the next lesson, we will connect this physical interpretation to the mathematical concept of phase shift and derive the relationship between time delay and phase difference.

Exercises

Exercise 1

Generate three sinusoidal signals with the following time delays:

- 0 ms

- 20 ms

- 40 ms

Keep the amplitude, frequency, and initial phase constant.

Describe how the waveform changes as the time delay increases.

Exercise 2

A sinusoidal signal has the following parameters:

- Amplitude = 1

- Frequency = 5 Hz

- Initial Phase = 0°

Generate delayed versions of the signal using:

- 10 ms

- 30 ms

- 50 ms

Compare the resulting waveforms and explain your observations.

Exercise 3

A radio signal propagates through free space.

Receiver A detects the signal immediately.

Receiver B detects the same signal 25 ms later.

Answer the following questions:

- Which receiver is closer to the transmitter?

- Does the delayed signal have a different frequency?

- Does the delayed signal have a different amplitude?

- What characteristic of the waveform has changed?

Exercise 4

Explain the difference between the following concepts:

- Initial Phase

- Time Delay

Can two signals have the same initial phase but different time delays?

Can two signals have different initial phases but arrive at the same time?

Discuss your answers.

Exercise 5

Using MATLAB, investigate how increasing the time delay affects the waveform while keeping all other parameters constant.

Summarize your observations in your own words.

Key Takeaways

After completing this lesson, the following concepts should be clear:

- Time delay represents the difference in signal arrival time.

- The SI unit of time delay is the second (s).

- Time delay shifts a waveform horizontally along the time axis.

- Time delay does not change:

  - Amplitude

  - Frequency

  - Period

  - Waveform shape

- Time delay is a consequence of wave propagation.

- Every propagating wave requires time to travel through space.

- Different receivers generally observe the same waveform at different times.

- Time delay forms the physical foundation of antenna arrays and beamforming.

Remember:

Phase describes where a waveform begins.

Time delay describes when a waveform arrives.

Although both produce horizontal shifts in the time domain, they represent different physical concepts.

References

The concepts presented in this lesson are based on standard textbooks in signal processing, communications, electromagnetics, and antenna theory.

Textbooks

- Alan V. Oppenheim and Alan S. Willsky, Signals and Systems, 2nd Edition, Pearson Education.

- Simon Haykin and Barry Van Veen, Signals and Systems, 2nd Edition, Wiley.

- John G. Proakis and Dimitris G. Manolakis, Digital Signal Processing: Principles, Algorithms, and Applications, 4th Edition, Pearson.

- Constantine A. Balanis, Antenna Theory: Analysis and Design, 4th Edition, Wiley.

- Steven W. Smith, The Scientist and Engineer's Guide to Digital Signal Processing, California Technical Publishing.

Conclusion

In this lesson, we introduced the concept of time delay, a fundamental physical quantity that describes when a waveform arrives at an observation point after propagating through space.

Unlike the initial phase, which specifies the mathematical starting point of a sinusoidal waveform, time delay arises from the finite speed at which waves travel. Through mathematical analysis and MATLAB simulations, we demonstrated that introducing a time delay shifts the waveform along the time axis without changing its amplitude, frequency, period, or overall shape.

This distinction is particularly important because time delay is not simply another waveform parameter; it is a direct consequence of wave propagation. Whether the wave is electromagnetic, acoustic, or mechanical, different observation points generally receive the same waveform at different times.

The MATLAB simulation reinforced this physical interpretation by comparing several delayed waveforms generated from the same sinusoidal signal. The results clearly showed that increasing the delay postpones the arrival of the waveform while preserving all of its intrinsic characteristics.

Most importantly, this lesson establishes the physical foundation of beamforming. In an antenna array, signals arriving from a distant source do not reach every antenna element simultaneously. Instead, each antenna receives the wave with a slightly different propagation delay. These tiny differences in arrival time contain information about the direction of the incoming wave and form the basis of direction finding, steering vectors, and electronic beam steering.

In the next lesson, we will build directly upon this foundation by examining the relationship between time delay and phase shift. Although these two concepts often produce similar waveform shifts in the time domain, they are fundamentally different. We will derive the mathematical relationship between them and explain why a given time delay corresponds to different phase shifts at different frequencies.