Lesson 07 Report

Phase Shift vs. Time Delay

Project: Beamforming From First Principles Using MATLAB

Chapter: 01 – Fundamentals of Waves

Lesson: 07 – Phase Shift vs. Time Delay

Version: 1.0.0

Question

Are phase shift and time delay the same physical phenomenon?

If both appear as horizontal shifts of a sinusoidal waveform, why are they treated as different concepts in communication systems, radar, and beamforming?

How are these two quantities mathematically related?

Objective

After completing this lesson, students will be able to:

- Distinguish between phase shift and time delay.

- Explain the physical meaning of each concept.

- Derive the mathematical relationship between phase shift and time delay.

- Understand why phase depends on frequency while time delay does not.

- Generate equivalent phase-shifted and time-delayed signals using MATLAB.

- Interpret the similarities and differences between these two waveform transformations.

- Recognize why this relationship is fundamental in antenna arrays and beamforming.

Background

In the previous two lessons, we introduced two concepts that both appear to shift a sinusoidal waveform along the time axis.

In Lesson 05, we studied the initial phase, which determines the starting position of a waveform.

In Lesson 06, we examined time delay, which represents the difference in signal arrival time caused by wave propagation.

At first glance, these two effects seem identical.

In both cases, the waveform appears to move horizontally along the time axis while maintaining the same amplitude and frequency.

This visual similarity often leads beginners to believe that phase shift and time delay are exactly the same.

However, they describe two fundamentally different ideas.

A phase shift is a mathematical description of where a periodic waveform begins within its cycle. It is measured as an angle and expressed in degrees or radians.

A time delay, on the other hand, is a physical quantity measured in seconds. It describes the amount of time required for a waveform to travel from one location to another or the difference in arrival times between two observations of the same signal.

Although these concepts originate from different physical interpretations, they become mathematically connected when the waveform has a fixed frequency.

For a sinusoidal signal, a specific time delay corresponds to a specific phase shift. Likewise, a particular phase shift can be represented as an equivalent time delay.

This equivalence is valid only for a given frequency.

If the frequency changes while the time delay remains constant, the corresponding phase shift also changes.

This important observation demonstrates that time delay is frequency-independent, whereas phase shift is frequency-dependent.

Understanding this distinction is essential for advanced topics such as wave propagation, steering vectors, direction-of-arrival estimation, and beamforming, where physical propagation delays are often represented mathematically as phase differences.

Mathematics

The general expression of a sinusoidal signal is

x(t)=Asin⁡(2πft+ϕ),

where:

- Ais the amplitude,

- fis the frequency,

- tis time,

- ϕis the phase angle.

A delayed sinusoidal signal can also be written as

x(t)=Asin⁡ ⁣2πf(t-τ),

where:

- τis the time delay.

Expanding the delayed signal gives

x(t)=Asin⁡(2πft-2πfτ).

Comparing this expression with the standard sinusoidal equation immediately reveals the relationship

ϕ=2πfτ.

Equivalently,

τ=ϕ2πf.

These equations show that phase shift and time delay are mathematically equivalent only after the signal frequency is specified.

For a fixed frequency, either quantity can be converted into the other.

However, because frequency appears explicitly in the equations, changing the frequency changes the corresponding phase shift for the same time delay.

This relationship forms one of the most important mathematical foundations of beamforming and array signal processing.

MATLAB Implementation

The objective of this MATLAB simulation is to compare phase shift and time delay under identical signal conditions and to demonstrate their mathematical relationship.

To ensure that only the concepts of interest are investigated, the following parameters remain constant throughout the simulation:

- Amplitude

- Frequency

- Sampling frequency

- Simulation duration

A sinusoidal signal is first generated without any phase shift or time delay. This waveform serves as the reference signal.

Next, a second waveform is generated by introducing an initial phase shift of 90°.

Finally, a third waveform is generated by applying the equivalent time delay calculated from

τ=ϕ2πf.

The reusable MATLAB function generateSineWave.m is used to generate all three waveforms.

The three signals are then plotted on the same figure to allow direct visual comparison.

Unlike the previous lessons, the objective here is not simply to observe a waveform parameter, but to compare two different mathematical descriptions of waveform displacement.

This comparison prepares students to understand why phase differences are frequently used instead of propagation delays in communication systems and beamforming algorithms.

Visualization

The generated figure displays three sinusoidal waveforms:

- Original signal

- Phase-shifted signal

- Time-delayed signal

For the selected frequency, the phase-shifted waveform and the time-delayed waveform almost perfectly overlap.

This observation confirms the mathematical relationship derived earlier.

For a fixed frequency,

ϕ=2πfτ,

a specific phase shift corresponds to one unique time delay.

The visualization also demonstrates that both transformations produce the same horizontal displacement of the waveform when the frequency remains constant.

However, this agreement should not lead to the conclusion that phase shift and time delay are identical concepts.

The figure represents only one operating frequency.

If the frequency were changed while keeping the time delay constant, the equivalent phase shift would also change.

Therefore, the overlap observed in this simulation is a consequence of the selected frequency rather than a universal property.

Discussion

This lesson highlights one of the most important conceptual distinctions in signal processing and beamforming.

Although a phase shift and a time delay can produce nearly identical waveforms for a fixed frequency, they represent fundamentally different quantities.

A phase shift is an angular quantity.

It specifies the position of a waveform within one oscillation cycle and is measured in degrees or radians.

A time delay is a temporal quantity.

It specifies how long a waveform requires to reach a particular observation point and is measured in seconds.

The mathematical relationship

ϕ=2πfτ

connects these two quantities.

However, this equation also reveals an important limitation.

Because frequency appears explicitly in the equation, the same time delay corresponds to different phase shifts at different frequencies.

For example, consider a propagation delay of one millisecond.

At a low frequency, this delay may correspond to only a small phase shift.

At a much higher frequency, the same physical delay may represent several complete cycles.

Consequently, time delay is an absolute physical quantity, whereas phase shift is always interpreted relative to the signal frequency.

This distinction is particularly important in beamforming.

When an electromagnetic wave reaches an antenna array, each antenna element receives the signal with a slightly different propagation delay.

These delays originate from geometry and wave propagation.

Instead of processing these delays directly, many beamforming algorithms convert them into phase differences using

ϕ=2πfτ.

Representing propagation delays as phase shifts greatly simplifies the mathematical formulation of steering vectors, array manifolds, and adaptive beamforming algorithms.

For this reason, the relationship developed in this lesson is not merely a mathematical identity.

It serves as the bridge between the physical behavior of propagating waves and the mathematical models used throughout modern array signal processing.

Exercises

Exercise 1

Generate three sinusoidal signals using the following conditions:

- Original signal

- Phase shift = 45°

- Equivalent time delay

Plot all three waveforms on the same figure and compare the results.

Exercise 2

Repeat the simulation using the following phase shifts:

- 30°

- 60°

- 90°

- 180°

For each case:

- Calculate the equivalent time delay.

- Verify the result using MATLAB.

- Compare the phase-shifted and time-delayed waveforms.

Exercise 3

A sinusoidal signal has a frequency of 10 Hz.

Calculate the equivalent time delay for a phase shift of:

- 45°

- 90°

- 180°

Verify your calculations numerically and graphically.

Exercise 4

Assume that a signal experiences a propagation delay of

τ=5 ms.

Calculate the corresponding phase shift for the following frequencies:

| Frequency | Phase Shift |
| --- | --- |
| 5 Hz |  |
| 10 Hz |  |
| 20 Hz |  |
| 50 Hz |  |

Discuss why the phase shift changes while the physical time delay remains constant.

Exercise 5

Answer the following conceptual questions.

- Can two signals have the same time delay but different phase shifts?

- Can two signals have the same phase shift but different time delays?

- Which quantity is determined by wave propagation?

- Which quantity depends on signal frequency?

Explain your reasoning.

Key Takeaways

After completing this lesson, the following concepts should be clear:

- Phase shift and time delay are not the same physical quantity.

- Phase shift is measured in degrees or radians.

- Time delay is measured in seconds.

- A phase shift describes the position of a waveform within one cycle.

- A time delay describes when a waveform arrives.

- For a sinusoidal signal, these quantities are related by

ϕ=2πfτ

and

τ=ϕ2πf.

- Time delay is independent of frequency.

- Phase shift depends directly on frequency.

- Beamforming algorithms frequently convert propagation delays into phase differences to simplify signal processing.

Remember:

Time delay belongs to the physical world.

Phase shift belongs to the mathematical description of periodic signals.

They are connected by frequency, but they are not interchangeable physical concepts.

References

The concepts presented in this lesson are based on standard textbooks in signal processing, communications, electromagnetics, and antenna array processing.

Textbooks

- Alan V. Oppenheim and Alan S. Willsky, Signals and Systems, 2nd Edition, Pearson Education.

- Simon Haykin and Barry Van Veen, Signals and Systems, 2nd Edition, Wiley.

- John G. Proakis and Dimitris G. Manolakis, Digital Signal Processing: Principles, Algorithms, and Applications, 4th Edition, Pearson.

- Constantine A. Balanis, Antenna Theory: Analysis and Design, 4th Edition, Wiley.

- Harry L. Van Trees, Optimum Array Processing, Wiley.

Conclusion

In this lesson, we examined one of the most important conceptual relationships in signal processing: the connection between phase shift and time delay.

Although both quantities produce similar horizontal displacements of a sinusoidal waveform, they originate from different physical principles. A phase shift describes the position of a waveform within one oscillation cycle, whereas a time delay describes the arrival time of a waveform after propagating through space.

By deriving the equations

ϕ=2πfτ

and

τ=ϕ2πf,

we established the mathematical relationship between these two concepts. The MATLAB simulation demonstrated that, for a fixed frequency, a phase-shifted waveform and its equivalent time-delayed waveform overlap almost perfectly. However, we also showed that this equivalence depends on the signal frequency. When the frequency changes, the same physical time delay corresponds to a different phase shift.

This distinction is fundamental in engineering practice. Physical systems naturally introduce propagation delays, while signal processing algorithms often represent these delays as phase differences because phase-domain calculations are more convenient for periodic signals.

The concepts introduced in this lesson provide the mathematical bridge between wave propagation and array signal processing. They explain why beamforming algorithms, steering vectors, and direction-of-arrival estimation are commonly expressed in terms of phase differences even though the underlying physical phenomenon is propagation delay.

In the next lesson, we will introduce angular frequency, a quantity that further simplifies the mathematical representation of sinusoidal signals and serves as another essential building block for Fourier analysis, complex exponentials, and beamforming theory.