Lesson 11 Report

What Is Propagation Speed?

Project: Beamforming From First Principles Using MATLAB

Chapter: 01 – Fundamentals of Waves

Lesson: 11 – What Is Propagation Speed?

Version: 1.0.0

Question

What is propagation speed?

Why do different types of waves travel at different speeds?

How does propagation speed affect wavelength while leaving the frequency unchanged?

Objective

After completing this lesson, students will be able to:

- Define propagation speed.

- Explain the relationship between propagation speed, frequency, and wavelength.

- Distinguish between the properties of the wave source and the propagation medium.

- Calculate wavelength for different propagation speeds.

- Understand why electromagnetic waves travel at the speed of light in free space.

- Prepare for the study of wavefronts, plane waves, and antenna arrays.

Background

In the previous lesson, we learned that waves propagate through space by transporting energy and information. We also introduced the travelling-wave equation, which describes how a wave evolves in both time and space.

A natural question now arises:

How fast does a wave travel?

The answer depends on the type of wave and the medium through which it propagates.

For example, sound waves travel through air much more slowly than electromagnetic waves travel through free space.

Similarly, light travels more slowly in water than in air.

This property is called the propagation speed (or wave velocity).

Propagation speed determines how quickly a disturbance moves from one location to another.

It is important to distinguish between the motion of the wave and the motion of the medium itself.

When sound propagates through air, individual air molecules only oscillate around their equilibrium positions. The molecules do not travel from the loudspeaker to the listener.

Likewise, electromagnetic waves propagate through changing electric and magnetic fields rather than by transporting physical matter.

One of the most common misconceptions is that changing the propagation medium changes the frequency of the wave.

In reality, the frequency is determined by the source.

When a wave enters a different medium, its frequency remains constant, while its propagation speed and wavelength change.

This principle is fundamental in optics, communications, radar, and antenna engineering.

Mathematics

Propagation speed is commonly represented by

v

and is measured in

m/s.

The relationship between propagation speed, frequency, and wavelength is

v=fλ

where

- vis the propagation speed,

- fis the frequency,

- λis the wavelength.

This equation is one of the most important relationships in wave physics.

It shows that three quantities are always connected.

If any two are known, the third can be calculated.

Rearranging the equation gives

λ=vf

and

f=vλ

These equations explain an important physical principle.

If the frequency remains constant while the propagation speed increases, the wavelength must also increase.

Conversely, if the propagation speed decreases, the wavelength becomes shorter.

For electromagnetic waves propagating in free space,

v=c≈3×108 m/s

where cis the speed of light in vacuum.

This value is essentially constant for all radio, microwave, and optical waves travelling through free space.

However, when electromagnetic waves propagate through materials such as glass, water, or plastic, the propagation speed decreases.

Since the source continues to generate the same frequency, the wavelength becomes shorter inside the material.

This behaviour is responsible for many optical phenomena, including refraction and lens operation.

MATLAB Implementation

The objective of this MATLAB simulation is to investigate how propagation speed influences the spatial characteristics of a wave while the frequency remains constant.

To isolate the effect of propagation speed, the simulation uses:

- A constant amplitude

- A constant frequency

- Three different propagation speeds

For each propagation speed, the wavelength is calculated using

λ=vf,

where vis the propagation speed and fis the signal frequency.

The travelling-wave equation

xzt=Asin⁡ωt-kz

is then evaluated at a fixed instant in time.

Since the frequency is identical for all three cases, any differences observed in the generated waveforms are caused solely by changes in propagation speed.

The calculated wavelength corresponding to each propagation speed is displayed in both the figure legend and the MATLAB Command Window.

This implementation provides a clear numerical and graphical demonstration of the relationship between propagation speed and wavelength.

Visualization

The generated figure compares three travelling waves propagating with different velocities.

Although all three waves have the same frequency, they exhibit different wavelengths.

The wave with the lowest propagation speed has the shortest wavelength.

Its oscillations are packed closely together, producing more cycles over the same distance.

As the propagation speed increases, the wavelength increases proportionally.

Consequently, the distance between adjacent crests becomes larger.

An important observation is that the number of oscillations generated each second remains unchanged.

Only the physical distance occupied by one complete cycle changes.

This illustrates that propagation speed influences the spatial behaviour of the wave rather than the oscillation rate established by the source.

The visualization therefore reinforces one of the most fundamental principles of wave physics:

Changing the propagation speed changes the wavelength, but it does not change the frequency.

Discussion

One of the greatest sources of confusion for students studying waves is determining which quantities are controlled by the source and which are determined by the medium.

The signal source—such as a radio transmitter, loudspeaker, or function generator—determines the frequency of the emitted wave.

Once generated, that frequency remains fixed unless the source itself changes.

The propagation medium, however, determines how quickly the wave travels.

For sound waves, the propagation speed depends on properties such as temperature, density, and elasticity of the medium.

For electromagnetic waves, the propagation speed depends on the electrical properties of the material, particularly its permittivity and permeability.

Because the propagation speed can change while the source frequency remains constant, the wavelength must adjust accordingly.

This relationship follows directly from

v=fλ.

Consider a beam of light entering water from air.

The source continues to emit light at exactly the same frequency.

However, because light travels more slowly in water, its wavelength decreases.

When the light exits the water and returns to air, the propagation speed increases again, and the wavelength returns to its original value.

The frequency remains unchanged throughout this entire process.

This behaviour is fundamental to many engineering applications.

Optical fibres guide light because the propagation speed changes between different materials.

Radar waves travel through the atmosphere with nearly the speed of light but slow slightly in rain or other dense media.

Acoustic imaging systems rely on different sound propagation speeds in various biological tissues.

In antenna engineering, propagation speed determines the wavelength associated with a given operating frequency.

Since antenna dimensions and antenna spacing are typically expressed in fractions of the wavelength, understanding propagation speed is essential for correctly designing antenna arrays.

For beamforming, this concept becomes especially important.

Beamforming algorithms estimate or control the phase relationships between signals arriving at multiple antennas.

Those phase relationships depend directly on wavelength, and wavelength depends directly on propagation speed.

Thus, although propagation speed may appear to be a simple physical quantity, it ultimately influences array geometry, beam steering, angular resolution, and direction-of-arrival estimation.

Exercises

Exercise 1

Using the relationship

v=fλ,

calculate the wavelength for each of the following cases.

| Propagation Speed (m/s) | Frequency (Hz) | Wavelength (m) |
| --- | --- | --- |
| 20 | 2 |  |
| 40 | 2 |  |
| 60 | 2 |  |
| 80 | 2 |  |

Exercise 2

An electromagnetic wave has a frequency of

2.4 GHz.

Calculate its wavelength in:

- Free space

- A medium where the propagation speed is 2.0×108m/s

Compare the two results and explain why they are different.

Exercise 3

Modify the MATLAB simulation.

Keep the frequency constant while changing the propagation speed.

Try the following values:

- 1 m/s

- 3 m/s

- 5 m/s

- 10 m/s

Observe how the waveform changes and explain your observations.

Exercise 4

Suppose a transmitter generates a sinusoidal signal with a frequency of

100 MHz.

The wave propagates through three different media.

| Medium | Propagation Speed |
| --- | --- |
| Medium A | 3×108m/s |
| Medium B | 2.4×108m/s |
| Medium C | 1.5×108m/s |

Calculate the wavelength in each medium.

Which quantity remains unchanged?

- Frequency

- Propagation speed

- Wavelength

Explain your answer.

Exercise 5

Answer the following conceptual questions.

- What determines the frequency of a wave?

- What determines the propagation speed?

- Why does wavelength change when a wave enters a different medium?

- Why does the frequency remain unchanged?

- Why is propagation speed important in antenna engineering?

Key Takeaways

After completing this lesson, the following concepts should be clear:

- Propagation speed describes how fast a wave travels through space.

- Propagation speed is measured in metres per second (m/s).

- The fundamental relationship between wave quantities is

v=fλ

- Frequency is determined by the source.

- Propagation speed is determined by the medium.

- Wavelength depends on both frequency and propagation speed.

- Changing the propagation medium changes the propagation speed and wavelength, but not the frequency.

- For electromagnetic waves in free space,

c≈3×108 m/s

is the propagation speed.

Remember:

The source determines the frequency.

The medium determines the propagation speed.

The wavelength adjusts automatically to satisfy

v=fλ.

This single relationship connects the three most important quantities in wave physics.

References

The concepts presented in this lesson are based on standard textbooks in electromagnetics, wave physics, antenna theory, and communication systems.

Textbooks

- Constantine A. Balanis, Antenna Theory: Analysis and Design, 4th Edition, Wiley.

- David K. Cheng, Field and Wave Electromagnetics, 2nd Edition, Addison-Wesley.

- Matthew N. O. Sadiku, Elements of Electromagnetics, 7th Edition, Oxford University Press.

- Alan V. Oppenheim and Alan S. Willsky, Signals and Systems, 2nd Edition, Pearson.

- John G. Proakis and Masoud Salehi, Fundamentals of Communication Systems, Pearson.

Conclusion

In this lesson, we introduced propagation speed, the physical quantity that describes how rapidly a wave travels through space.

Building upon the previous lessons, we completed the fundamental relationship between propagation speed, frequency, and wavelength through the equation

v=fλ.

This equation demonstrates that the three quantities are inseparably linked. While the frequency is determined by the source that generates the wave, the propagation speed is determined by the medium through which the wave travels. As a result, any change in propagation speed is accompanied by a corresponding change in wavelength, while the frequency remains constant.

The MATLAB simulation illustrated this principle by comparing waves with identical frequencies propagating at different speeds. The results showed that faster propagation produces longer wavelengths, whereas slower propagation results in shorter wavelengths.

Understanding this relationship is essential because wavelength serves as the natural unit of distance in antenna engineering. Since antenna dimensions, element spacing, and spatial phase differences all depend on wavelength, propagation speed indirectly influences the design and performance of antenna arrays.

With the concepts of frequency, wavelength, propagation, and propagation speed now established, we are ready to examine the geometric description of propagating waves. The next lesson introduces wavefronts, providing the conceptual bridge between travelling waves and the plane-wave models used throughout antenna theory and beamforming.