Lesson 14 Report

Spatial Phase Difference

Chapter 01 – Fundamentals of Waves

Lesson 14

Spatial Phase Difference

Project

Beamforming From First Principles Using MATLAB

Version

1.0.0

Question

Imagine two antennas placed a short distance apart.

A plane electromagnetic wave is traveling toward the antennas.

Both antennas receive exactly the same transmitted signal.

Yet, when we measure the received waveforms, we discover that the signals are not perfectly aligned in phase.

Why?

Where does this phase difference come from?

Did the transmitter generate two different signals?

Did the wave change while propagating?

Or is there another physical explanation?

These questions mark the beginning of one of the most important concepts in antenna array processing.

Many beamforming textbooks immediately introduce the spatial phase equation

Δϕ=2πλΔd

and continue with steering vectors and array factors.

Although mathematically correct, this approach often leaves students with a formula that is easy to memorize but difficult to understand.

In this lesson we take a different approach.

Instead of starting with mathematics, we begin by observing the wave itself.

We first examine a plane wave and identify its wavefront.

Next, we investigate the direction in which the wave propagates.

Then we place two antennas in its path and simply watch what happens.

From this observation, we gradually discover that

- the antennas are located at different positions,

- the wave reaches them at different times,

- this creates a propagation delay,

- and the delay naturally appears as a phase difference.

Only after this physical intuition has been established do we introduce the mathematical relationship describing the spatial phase difference.

By the end of this lesson, the reader will understand that the phase difference observed by an antenna array is not created by the transmitter.

Instead, it is a direct consequence of wave propagation through space.

This simple observation forms the physical foundation of beamforming, steering vectors, array factors, and direction-of-arrival estimation that will be studied throughout the remaining chapters.

Objective

The objective of this lesson is to develop an intuitive and physically meaningful understanding of spatial phase difference, one of the most fundamental concepts in antenna arrays and beamforming.

Rather than beginning with equations, this lesson begins with a simple physical observation.

A plane electromagnetic wave propagates through space and encounters two antennas positioned at different locations.

Although both antennas receive the same transmitted waveform, they do not receive it at the same instant.

The purpose of this lesson is to understand why this happens and how this simple geometric fact leads naturally to a phase difference between the received signals.

Throughout this lesson, the reader will investigate the following sequence of events:

- Observe a plane electromagnetic wave.

- Understand the direction of wave propagation.

- Place two antenna elements in the propagation path.

- Observe that the wavefront reaches one antenna before the other.

- Recognize that different propagation distances produce different arrival times.

- Understand that propagation time differences become phase differences for sinusoidal signals.

- Relate this spatial phase difference to the physical principle behind beamforming.

Instead of memorizing formulas, the reader is encouraged to understand the complete chain of physical events that produces the observed phase difference.

The lesson emphasizes that:

- the transmitted signal remains unchanged,

- the propagation medium remains unchanged,

- the only difference is the spatial location of the observing antennas.

This change in observation position alone is sufficient to create different received phases.

By the end of this lesson, the reader should be able to:

- Explain the difference between a wavefront and the direction of propagation.

- Describe how the arrival angle influences the propagation path to each antenna.

- Explain why two antennas observing the same wave can measure different phases.

- Calculate the propagation path difference between antenna elements.

- Compute the corresponding propagation time delay.

- Determine the resulting spatial phase difference.

- Interpret spatial phase difference using geometry, wave propagation, and phasor representation.

- Recognize that beamforming exploits these naturally occurring phase differences rather than creating them.

This lesson serves as the transition from the study of individual electromagnetic waves to the study of antenna arrays.

The physical intuition developed here becomes the foundation for the remaining chapters, including antenna arrays, beamforming, direction-of-arrival estimation, multiple-input multiple-output (MIMO) systems, and integrated sensing and communication (ISAC).

Background

In the previous lessons, we studied the fundamental properties of sinusoidal waves, including amplitude, frequency, period, wavelength, propagation speed, propagation delay, wavefronts, and the basic mechanisms of wave propagation.

Each of these concepts described the behavior of a wave from the perspective of a single observation point.

However, antenna arrays introduce a fundamentally different question.

Instead of observing a wave at one location, we observe the same wave simultaneously at multiple locations.

This change in perspective is the beginning of array signal processing.

Consider a plane electromagnetic wave propagating through free space.

A plane wave consists of infinitely large wavefronts, where every point on a single wavefront has exactly the same phase.

The wavefront itself does not move along its own surface.

Instead, the wave propagates in a direction that is perpendicular to the wavefront.

This propagation direction is commonly described by the arrival angle relative to the antenna array.

Now imagine placing two antenna elements along the array axis.

Because the antennas occupy different spatial positions, the incoming wavefront cannot reach both antennas at exactly the same moment unless it arrives perfectly perpendicular to the array.

For most arrival angles, one antenna is encountered first, while the other is reached slightly later.

Although this propagation delay is usually extremely small—often measured in nanoseconds—it has an important consequence.

A sinusoidal signal continuously changes its phase with time.

Therefore, even a tiny propagation delay produces a measurable phase difference between the received signals.

This phase difference is not introduced by the transmitter, nor is it caused by any change in the waveform during propagation.

It is simply the natural consequence of observing the same propagating wave from different locations in space.

This phenomenon is known as the spatial phase difference.

Every beamforming algorithm, steering vector, array factor, and direction-of-arrival estimation technique is ultimately built upon this simple physical principle.

For this reason, understanding the origin of spatial phase difference is considerably more valuable than memorizing the mathematical equations that describe it.

Once the physical mechanism becomes clear, the mathematical model follows naturally.

Mathematics

The mathematical model of spatial phase difference follows directly from the physical observations developed in the previous sections.

Consider two antenna elements separated by a distance (d).

Assume that a plane electromagnetic wave arrives from an angle (\theta), measured with respect to the array axis.

Because the antennas are located at different positions, the wavefront reaches the first antenna before reaching the second.

The additional propagation distance traveled by the wave is called the propagation path difference.

From elementary geometry,

Δd=dcosθ

where

- (d) is the spacing between adjacent antenna elements,

- (θ) is the arrival angle,

- (Δd) is the propagation path difference.

Since electromagnetic waves propagate at the speed of light (c), the corresponding propagation time delay is

Δt=Δdc

Substituting the path difference gives

Δt=dcosθc

A sinusoidal signal with frequency (f) completes one cycle every

T=1f

seconds.

Therefore, the fraction of one cycle represented by the propagation delay is

ΔtT

Since one complete cycle corresponds to (2\pi) radians, the resulting spatial phase difference becomes

Δϕ2πΔtT

Substituting

T=1f

gives

Δϕ2πfΔt

Finally,

Δt=dcosθc

yields

Δϕ=2πfdcosθc

Because

λ=cf

the equation can be written in its most common form,

Δϕ=2πλdcosθ

This equation is not merely a mathematical expression.

It summarizes the complete physical process developed throughout this lesson:

Arrival Angle→Path Difference→Time Delay→Spatial Phase Difference

Every beamforming algorithm introduced in the following chapters ultimately relies on this relationship.

MATLAB Implementation

The MATLAB implementation mirrors the physical reasoning presented in this lesson.

Instead of computing the phase difference directly, the simulation follows the same sequence experienced by the propagating wave.

The simulation performs the following steps:

- Generate a two-element antenna array.

- Display the incoming plane wave.

- Display the propagation direction.

- Position the antenna elements.

- Animate the incoming wavefront.

- Compute the propagation path difference.

- Compute the propagation time delay.

- Compute the spatial phase difference.

- Plot the received sinusoidal signals.

- Display the corresponding phasor representation.

- Present the numerical simulation results.

This workflow intentionally follows the physical propagation process rather than the mathematical derivation.

Consequently, the MATLAB simulation serves as a visual explanation of the theory rather than simply verifying an equation.

Visualization

The MATLAB simulation provides a visual interpretation of the physical concepts introduced throughout this lesson.

Instead of presenting only numerical calculations, the simulation illustrates the complete propagation process step by step.

The generated figures include:

Figure 1 — Plane Wave

A plane electromagnetic wave is displayed using parallel wavefronts. This figure introduces the concept of a wavefront, emphasizing that every point on the same wavefront has the same phase.

Figure 2 — Propagation Direction

The direction of propagation is illustrated using an arrow perpendicular to the wavefronts. This visualization clarifies that the arrival angle is defined with respect to the propagation direction rather than the wavefront itself.

Figure 3 — Antenna Array and Incoming Wave

Two antenna elements are placed in the path of the incoming plane wave. This figure illustrates the geometric relationship between the antenna positions and the arriving wave.

Figure 4 — Wave Arrival Animation

An animated wavefront propagates across the antenna array.

The animation demonstrates that:

- the wavefront reaches one antenna first,

- the second antenna is reached slightly later,

- the transmitted signal remains unchanged,

- the observed delay is solely due to propagation.

Figure 5 — Received Signals

The received sinusoidal signals are plotted together.

Although both signals have identical amplitudes and frequencies, one signal is shifted in time because of the propagation delay.

Figure 6 — Phasor Representation

The received signals are represented as phasors.

The angular separation between the phasors corresponds to the spatial phase difference calculated during the simulation.

Discussion

One of the most common misconceptions in introductory beamforming is the assumption that different antenna elements receive different transmitted signals.

The simulation presented in this lesson demonstrates that this assumption is incorrect.

Every antenna receives the same electromagnetic wave.

The difference lies only in the observation point.

Since each antenna occupies a different position in space, the propagating wave requires a different travel distance to reach each element.

This additional propagation distance introduces a propagation delay.

For sinusoidal signals, a propagation delay is equivalent to a phase shift.

Therefore, the spatial phase difference is not an arbitrary mathematical quantity but a direct consequence of wave propagation.

The lesson also highlights the distinction between two frequently confused concepts:

- Initial phase, which is determined by the signal source.

- Spatial phase difference, which is determined by the propagation geometry.

Understanding this distinction is essential before studying antenna arrays, steering vectors, beamforming algorithms, and direction-of-arrival estimation.

Exercises

- Explain why two antennas observing the same plane wave generally measure different phases.

- Describe the relationship between propagation path difference and propagation time delay.

- Derive the expression for the spatial phase difference starting from the propagation path difference.

- What would happen if the arrival angle were equal to 90°?

- What would happen if the antenna spacing were doubled while the arrival angle remained unchanged?

- Run the MATLAB simulation using several different arrival angles and explain how the phase difference changes.

- Modify the antenna spacing and observe its influence on the propagation delay and phase difference.

- Explain why beamforming relies on phase differences instead of amplitude differences.

Key Takeaways

The most important concepts learned in this lesson are summarized below.

- A plane wave propagates in a specific direction.

- Wavefronts are always perpendicular to the propagation direction.

- Different antenna positions create different propagation distances.

- Different propagation distances produce different propagation delays.

- Propagation delays produce spatial phase differences.

- Spatial phase difference is a physical consequence of wave propagation.

- Beamforming exploits naturally occurring spatial phase differences to control the directional response of an antenna array.

The complete physical chain can be summarized as

Arrival Angle

↓

Propagation Path Difference

↓

Propagation Time Delay

↓

Spatial Phase Difference

↓

Beamforming

References

- Balanis, C. A., Antenna Theory: Analysis and Design, 4th Edition, John Wiley & Sons, 2016.

- Balanis, C. A., Advanced Engineering Electromagnetics, 2nd Edition, John Wiley & Sons, 2012.

- Kraus, J. D., and Marhefka, R. J., Antennas: For All Applications, 3rd Edition, McGraw-Hill, 2002.

- Mailloux, R. J., Phased Array Antenna Handbook, 3rd Edition, Artech House, 2018.

- Van Trees, H. L., Optimum Array Processing, Part IV of Detection, Estimation, and Modulation Theory, John Wiley & Sons, 2002.

- Oppenheim, A. V., Willsky, A. S., and Nawab, S. H., Signals and Systems, 2nd Edition, Prentice Hall, 1997.

- Haykin, S., and Van Veen, B., Signals and Systems, 2nd Edition, John Wiley & Sons, 2003.

- Orfanidis, S. J., Electromagnetic Waves and Antennas, Rutgers University.

Conclusion

In this lesson, we investigated one of the most fundamental concepts underlying antenna arrays and beamforming: spatial phase difference.

Rather than beginning with mathematical equations, we followed the physical journey of an electromagnetic plane wave as it propagated toward two spatially separated antenna elements.

Through observation, geometry, mathematical analysis, and MATLAB simulation, we demonstrated that the phase difference measured by the antennas is a natural consequence of different propagation distances and arrival times.

This understanding forms the physical foundation for all subsequent topics in this course, including antenna arrays, beamforming, steering vectors, direction-of-arrival estimation, MIMO systems, and integrated sensing and communication.

With this foundation established, we are now prepared to study how multiple waves combine in space and how their interaction produces constructive and destructive interference—the subject of the next lesson.